# Literature Review — Payload-Aware Open-Set Interference Recognition for GEO Transponders

## Abbreviations and Definitions

Symbols are defined where first used; this list collects them. Signals are complex baseband.

| Symbol | Meaning | Unit |
| --- | --- | --- |
| x(t), y(t) | Composite uplink signal; downlink observation | – |
| sk(t), Pk, fk, θk(t) | k-th planned carrier: waveform, power, center frequency, phase noise | –, W, Hz, rad |
| i_m(t) | m-th interferer | – |
| nu(t), nd(t), N0 | Uplink and downlink noise; noise power spectral density | –, –, W/Hz |
| hIMUX, hOMUX | Input and output multiplexer impulse responses | – |
| 𝒢{·}, A(r), Φ(r) | TWTA operator; AM/AM and AM/PM characteristics | –, –, rad |
| αa, βa, αφ, βφ | Saleh model parameters | – |
| IBO, Pin,sat | Input back-off; input power at saturation | dB, W |
| fIM3, C/IM₃ | Third-order intermodulation frequency; carrier-to-IM3 ratio | Hz, dB |
| yco, yx, zco, zx | Co- and cross-polar observations; transponder outputs per polarization | – |
| κ, XPD | Polarization coupling coefficient; cross-polar discrimination | –, dB |
| ρ, C/I | Under-carrier power ratio, ρ = (C/I)⁻¹; carrier-to-interference ratio | –, dB |
| Sy(f), Sy^α(f), α, T_s | Power spectral density; spectral correlation function; cycle frequency; symbol period | W/Hz, –, Hz, s |
| T, γ, γi, αCFAR, Nr | Detection statistic; thresholds; CA-CFAR scale factor; number of reference cells | – |
| PFA, PD | False-alarm and detection probabilities | – |
| fk(x), z, μk, Σ, τ | Logits; embedding; class means; shared covariance; rejection threshold | – |
| S(x), TE | Open-set score; temperature of the energy score | – |
| CCR, FPR, OSCR | Correct classification rate; false-positive rate; open-set classification rate | – |
| Δpayload(c), M, 𝒟c | Payload gap at test condition c; metric; test set at c | – |

## 1. Introduction

Interference is a routine, recurring cost of operating geostationary (GEO) satellites, not a rare fault. A database of 3,650 incidents compiled by the Satellite Users Interference Reduction Group (SUIRG) attributed 62% of cases to human error, cross-polarization and equipment problems [1]. Operator figures tell a similar story: Intelsat has attributed up to 33% of its interference to cross-polarization leakage and 47% to unauthorized carriers [2], and the Interference Reduction Group (IRG) has estimated that VSAT terminals cause about 40% of all interference and half of the resulting downtime [3].

These statistics come from industry and trade sources rather than peer-reviewed measurement, but they agree on the main point. A small set of mechanisms dominates the problem: mispointed antennas, misconfigured equipment, cross-polar leakage and carriers transmitted where they should not be. Mispointing and misconfiguration alone are estimated to cover 70–80% of cases [4]. The two categories matter because they call for different responses. Unintentional interference is mostly handled operationally, through better terminal commissioning, carrier monitoring and identification of the interfering carrier, and it is typically resolved by contacting the offending operator. Intentional jamming adds to this picture; in 2026, sustained interference with on/off cycling was reported against at least six commercial GEO satellites [5]. Deliberate jamming cannot be resolved that way. It requires geolocation of the source, escalation through regulatory and diplomatic channels, and mitigation on the satellite or terminal side. Most GEO payloads are transparent bent-pipe repeaters, so they retransmit whatever arrives in the uplink band, and an operator cannot filter out interference at the satellite unless it differs from authorized traffic in frequency, polarization, direction or time. The reported use of on/off cycling and retargeting is also consistent with an attempt to make detection and geolocation harder, since the interfering signal is present only intermittently. For this reason, the 70–80% figure should not be read as a statement about risk to mission availability. It describes how frequent the incidents are, not how severe or how hard they are to mitigate.

Operators find most of these events through downlink spectrum monitoring. Carriers are measured against expected power levels and spectral masks, threshold alarms fire on deviations, and analysts inspect spectrum-analyzer traces to work out what happened [2], [6].

Even the machine-learning tools that have entered operator workflows still end in a fixed threshold on a deviation score [4]. Detecting that something is wrong is therefore the easier half of the task. The costly half is recognition: deciding which mechanism is at work, because each points to a different response — repointing a terminal, correcting a polarization setting, locating an unauthorized uplink, or reducing the payload drive.

This split matters because the observable symptoms of different mechanisms overlap. Cross-polarization leakage, adjacent-satellite interference and a carrier overlapping another carrier can all appear as a raised noise floor or degraded error rate on the same transponder, and an alarm raised on a spectrum trace does not say which of them is present [64], [65]. Intermodulation generated inside the payload amplifier can look similar to an external interferer, yet it is cured by changing the drive level rather than by finding a transmitter [55], [57]. Identification aids exist for the cases where the interferer cooperates, such as carrier-identification signalling for occasional-use earth stations [13], but they do not help with unauthorized or deliberate transmissions, which must be geolocated instead [5]. Learned detectors reported in the literature are mostly trained and evaluated on a few known interference types and on limited public datasets [20], [92], [93], so they do not by themselves solve recognition of events outside the training set, which is the open-set problem [68]. Recognition rather than detection is therefore where operator effort is spent and where the remaining research gap lies.

Two properties of the operator's observation make recognition harder than it first appears, and they shape this project. First, the monitoring station sees the downlink, after the uplink signal has passed through the bent-pipe transponder's input multiplexer (IMUX), traveling-wave tube amplifier (TWTA) and output multiplexer (OMUX). Detection on the ground is degraded by exactly this chain of transponder distortion and downlink noise [7], and the nonlinear amplifier creates intermodulation products that can resemble external interference. Second, not every event belongs to a known category: SUIRG reported that a further 34% of incidents came from unknown sources, often because they appeared and disappeared before they could be documented [1]. A recognizer forced to assign every event to a known class will mislabel exactly these cases.

This review is organized around those two properties. Section 2 covers classical monitoring and detection practice, and Section 3 the learning-based detectors and classifiers proposed for satellite, GNSS and general RF interference. Section 4 examines how the transponder has been modeled, and Section 5 asks which observables can separate the interference mechanisms of interest. Section 6 reviews open-set recognition, Section 7 evaluation under distribution shift, and Section 8 positions this project and its contributions against the literature.

## 2. Monitoring practice and classical detection

Classical interference detection treats the problem as hypothesis testing on received energy. The most developed line of work places a monitoring unit on board the satellite: the desired signal is decoded and subtracted, and an energy detector is applied to the residual [8]. Analyses of imperfect cancellation [8] and of weak interference [9] showed that the cancellation step is what makes low-power interference detectable at all. Pérez-Neira et al. framed the advantage of on-board detection as avoiding the downlink noise and transponder distortion that degrade detection on the ground [7]. For a ground operator this is the key observation: the monitored spectrum is not a clean view of the uplink.

With y[n] the received samples and ŝ[n] the reconstructed desired signal, the detector compares the residual energy with a threshold γ set for a target false-alarm probability:

T=1Nn=0N-1yn-ŝn2≷H0H1γ

Imperfect cancellation leaves part of the desired signal in T, which is the effect analyzed in [8].

Ground monitoring relies on the standard tools of spectral detection. Cell-averaging constant false-alarm rate (CA-CFAR) detection [10] sets adaptive thresholds against the local noise floor and is the natural baseline for spotting unexpected spectral peaks. Cyclostationary detection [11] exploits the periodic statistics of modulated signals; it is more robust at low SNR and can separate signals that overlap in frequency, at higher computational cost. Simpler schemes, such as tone detection within the subchannels of an on-board digital channelizer [12], target a single interference type. All of these methods answer whether energy is present where it should not be. None of them says what produced it.

For a spectral bin i with N_r reference cells R_i around it, CA-CFAR scales the mean reference power by a factor α_CFAR chosen for the design false-alarm probability. For square-law detection in exponentially distributed noise,

γi=η⋅1/Nrj∈RiPj,η=NrPFA-1/Nr-1

The cyclostationary counterpart is introduced in Section 5, where it matters most.

Identification has instead been pursued through cooperation and regulation. ITU-R Recommendation S.2062 defines a carrier identification (CID) system for occasional-use earth stations [13], and the DVB-CID metacarrier embeds a transmitter identity 27.5 dB below the host carrier's spectral density [14]. CID resolves the source of a carrier when the offending terminal transmits one, which is precisely what unauthorized or misconfigured equipment cannot be relied upon to do.

The hardest operational case, carrier-under-carrier interference, shows the limit of energy-based detection. When an unwanted carrier sits underneath an authorized one, the composite spectrum looks almost normal. Commercial monitoring systems — Rohde & Schwarz GSACSM [15], Integrasys VectorSat [16], Calian Decimator [17] and Kratos Monics [2] — all advertise carrier-under-carrier detection, and they achieve it by cancelling the known host carrier and inspecting the residual rather than by reading the power spectrum. Their performance claims are vendor statements without independent benchmarks. This practice already suggests that a learned recognizer cannot rely on the power spectrum alone for every class; Section 5 returns to this point.

## 3. Learning-based interference recognition

Deep learning entered satellite interference work as a way to replace hand-set thresholds with learned models of normal behavior. An early study trained a sparse autoencoder on real Ku-band DVB-S2 captures to flag interference by reconstruction error, and added an LSTM to classify digitally inserted terrestrial interferers such as LTE, UMTS and GSM [18]. A parallel approach trained an LSTM to predict the interference-free spectrum from real operator recordings and flagged deviations from that prediction [19]. In operational work with Eutelsat under ESA's ARTES program, an autoencoder detector of this kind reduced false detections by 44% on real data [6]. These systems learn what clean spectra look like, not what different interferers look like, so they detect but do not recognize.

The most active recent line, from SnT's SmartSpace project, learns from simulated interference. An on-board CNN localized NGSO interference to one of five subbands of a GSO uplink, with reduced-size FFT magnitudes giving the best accuracy [20]. On the ground side, a sequence of reconstruction-based detectors for NGSO-to-GSO downlink interference [21]–[23] culminated in a transformer detector (TrID) that improved detection accuracy over an energy detector by up to 31.23%, with time-domain inputs outperforming spectral ones [23]. The 2026 Agon framework adds masked-autoencoder pre-training, higher-order-statistics attention and a multi-task head, reaching an AUC of 0.9327 on the public NGSO–GSO dataset against 0.8318 for TrID-like baselines [24]. Closed-set classifiers separate single-tone, narrowband, broadband-noise, pulse and sweep interference in simulated AWGN channels with high accuracy [25]–[27], and on-board studies show such models can run on space-grade AI accelerators [28]. Surveys of AI for satellite communications count interference management among the most developed applications [29]–[31].

Across this body of work the signal model is the same: interferers are added linearly to a single desired carrier, in a single polarization, observed without the transponder in between, and the task is binary detection or closed-set classification. This is a reasonable model for NGSO coexistence at a user terminal, but it is not the situation of a GEO operator monitoring a loaded transponder. None of these studies includes multicarrier loading, amplifier nonlinearity, multiplexer filtering, a second polarization or the operator's carrier plan, and none tests whether a model can reject an interferer it has never seen.

The closest methodological precedent is in GNSS. Fraunhofer IIS has built a sustained program of interference classification on real recordings: multimodal fusion of spectral and time-domain inputs with Monte Carlo dropout as a reliability measure [32], low-cost monitoring stations [33], few-shot learning for novel interference classes [34], and evaluation under real-world discrepancies between controlled and field data [35]. Related work disentangles interference factors with variational autoencoders [36], releases fully simulated datasets with documented generators [37], and classifies jamming observed from terrestrial and LEO stations [38]. GNSS interference differs physically from the GEO case — the desired signal lies below the noise floor, and the interferer catalogue is dominated by chirp and pulsed sources — but the GNSS literature sets the evaluation standard we adopt: condition-based splits, explicit novel-class protocols and uncertainty-aware rejection.

General RF deep learning supplies the model building blocks. Over-the-air experiments with RadioML established that residual networks classify radio signals well, and that simulation-trained models degrade on real captures [39]. TorchSig's Sig53 [40] and WidebandSig53 [41] moved recognition from narrowband snippets to wideband spectrograms containing many simultaneous signals, the representation closest to a transponder monitoring view. Beyond end-to-end learning from spectrum data [42], two results bear directly on our design: CNNs trained on spectral-correlation-function inputs outperform raw-signal networks on real captures [43], and cyclostationary features improve the generalization of deep modulation classifiers [44]. Both motivate the cyclic-feature branch. On model size, small CNNs have been shown to suffice for interference classification [45], and time–frequency representations work well for interference in frequency-hopping systems [46]. A survey of deep-learning interference suppression also identifies open-set and out-of-distribution recognition as under-studied [47]. Residual networks [48], spectrogram transformers [49] and feature-wise conditioning [50] provide the standard mechanisms for a plan-conditioned recognizer.

## 4. The transponder as part of the observation

In a bent-pipe GEO transponder, every carrier the operator monitors has passed through the same nonlinear chain. The IMUX selects the transponder band, the TWTA amplifies the composite signal near saturation, and the OMUX filters the result before downlink. The Saleh model [51] remains the standard description of TWTA amplitude-to-amplitude (AM/AM) and amplitude-to-phase (AM/PM) conversion, with the operating point set by the input back-off (IBO). The DVB-S2 system design [52] and its accompanying modem studies [53] established the IMUX–TWTA–OMUX chain, phase-noise masks and predistortion as the reference satellite channel. The DVB-S2X implementation guidelines [54] extend this with tabulated curves for conventional and linearized TWTAs and a multicarrier scenario in which six carriers share one amplifier. This is the reference to which our simulator is made traceable.

Figure 1 · observation chain assumed in this project

Figure 1. Every uplink carrier and interferer reaches the monitoring station only after the IMUX, TWTA and OMUX, in both polarizations; the recognizer sees four views of the result.

In compact form, the composite uplink with K planned carriers and M interferers is

x(t) = \sum_{k=1}^{K} \sqrt{P_k}\, s_k(t)\, e^{\,j\left(2\pi f_k t + \theta_k(t)\right)} + \sum_{m=1}^{M} i_m(t) + n_u(t)

where s_k(t) is a pulse-shaped DVB-S2 waveform, θ_k(t) is phase noise and n_u(t) is uplink noise. The downlink observation is

y(t) = \left(h_{\mathrm{OMUX}} * \mathcal{G}\left\{h_{\mathrm{IMUX}} * x\right\}\right)(t) + n_d(t)

where the memoryless TWTA operator acts on the envelope of its input u(t):

\mathcal{G}\{u\}(t) = A\big(|u(t)|\big)\, e^{\,j\left(\arg u(t) + \Phi(|u(t)|)\right)}, \qquad A(r) = \frac{\alpha_a r}{1+\beta_a r^2}, \qquad \Phi(r) = \frac{\alpha_\phi r^2}{1+\beta_\phi r^2}

The output amplitude peaks at r_sat = 1/√β_a, and the operating point is set by the input back-off

\mathrm{IBO} = 10\log_{10}\frac{P_{\mathrm{in,sat}}}{P_{\mathrm{in}}}\ \mathrm{dB}

Figure 2 shows what these two curves imply for a single carrier.

Figure 2 · computed from the Saleh model (αa = 2.1587, βa = 1.1517, αφ = 4.0033, βφ = 9.1040)

Figure 2. Between 10 dB and 1 dB of back-off, gain compression grows from under 1 dB to about 5 dB and phase rotation doubles. A recognizer trained at one operating point therefore sees a differently distorted spectrum at another, which is the shift that C3 measures.

What matters for recognition is that the transponder creates spectral structure of its own. When several carriers share a TWTA, third-order intermodulation products fall at predictable frequencies such as 2f₁ − f₂, and their level rises faster than the carriers as drive increases [55]. The multiplexer filters add group delay that interacts with the amplifier: in a 36 MHz multicarrier DVB-S2 transponder, IMUX/OMUX group delay masks the TWTA characteristic seen at the output [56]. Noise power ratio measurements of space TWTAs quantify how much intermodulation fills the band under loading [57], and multicarrier operation also causes power robbing and suppression of weak carriers [58]. Seen from the monitoring station, payload intermodulation is therefore a structured, plan-dependent spectral signature that sits alongside, and can be confused with, external interference.

For carriers at f_1, …, f_K, the third-order products fall at

f_{\mathrm{IM3}} \in \left\{\, 2f_i - f_j,\; f_i + f_j - f_k \,\right\}, \qquad i, j, k \in \{1, \dots, K\}\ \text{distinct}

Well below saturation, third-order power grows by 3 dB for each decibel of drive while the carriers grow by 1 dB, so the carrier-to-intermodulation ratio improves by about 2 dB per dB of back-off [55], [58]:

P_{\mathrm{IM3}} \approx 3P_{\mathrm{in}} + c \quad\Rightarrow\quad \frac{\partial\,(C/\mathrm{IM}_3)}{\partial\,\mathrm{IBO}} \approx 2\ \mathrm{dB/dB}

Near saturation the slope flattens. Figure 3, simulated for this review, shows both effects in a three-carrier transponder: the regrowth peaks sit at the frequencies the plan predicts, and 9 dB of extra back-off lowers them by about 10 dB rather than 18 dB.

Figure 3 · simulated for this review: 3 QPSK carriers at 0.05 fs, RRC roll-off 0.2, Saleh TWTA, Welch PSD

Figure 3. Intermodulation is a plan-dependent signature: a recognizer that knows f_1, …, f_K can predict where it appears, and one that observes the response to back-off can confirm it. External interference obeys neither rule.

The satellite literature treats this chain as an impairment to compensate, not as context for recognition. Volterra equalization of multicarrier intermodulation [55], learned impairment compensation over TWTA and multiplexer models [59] and predistortion [53] all model the transponder carefully, but toward a different goal. Interference-recognition studies leave it out (Section 3). The omission has been noted in passing: a recent anti-jamming study observes that existing methods lack modeling of the distortion caused by transponder nonlinearity [60], though its own focus is suppression rather than recognition.

Measured amplifier data is harder to obtain than models. OpenDPD [61], [62] provides measured input–output data from a wideband power amplifier together with a PyTorch framework for behavioral modeling. It is a solid-state device, not a TWTA, so it cannot serve as ground truth for a GEO payload. It is useful instead as a source of model mismatch: a recognizer trained on Saleh-model data can be tested against a different, measured nonlinearity. Work on transmitter identification has shown that randomized amplifier nonlinearity models can be generated at scale for deep learning [63], which supports randomizing payload parameters during data generation rather than fixing a single transponder.

## 5. Interference mechanisms and identifiability

The interference classes in this project are not equally easy to tell apart, and the literature suggests that some cannot be separated from a single-polarization power spectrum at all. The main confusable groups are considered in turn, since they determine which observables the recognizer needs.

Cross-polarization leakage is among the largest single causes of interference in operator statistics [2]. It arises when a feed is misaligned, when rain depolarization reduces the cross-polar discrimination (XPD), or when antenna XPD is insufficient for a dual-polarized system [64], [65]. In a co-polar spectrum, leaked energy looks like any other unexpected carrier. It becomes identifiable only when the opposite polarization is observed, because leakage reproduces a carrier that exists on the other polarization at a level set by the XPD. Notably, the DVB-S2X reference channel assumes cross-polar and co-channel contributions to be negligible [54]. A dual-polarization observation therefore extends the reference model rather than following it, and needs its own grounding in XPD measurements [64], [65].

We model the two received polarizations as

\begin{bmatrix} y_{\mathrm{co}} \\ y_{\mathrm{x}} \end{bmatrix} = \begin{bmatrix} 1 & \kappa \\ \kappa & 1 \end{bmatrix} \begin{bmatrix} z_{\mathrm{co}} \\ z_{\mathrm{x}} \end{bmatrix} + \begin{bmatrix} n_{\mathrm{co}} \\ n_{\mathrm{x}} \end{bmatrix}, \qquad \mathrm{XPD} = -20\log_{10}|\kappa|\ \mathrm{dB}

where z_co and z_x are the transponder outputs of Section 4 on each polarization. Leakage into the co-polar channel is then a scaled copy of z_x, which can be tested by correlating the two channels; without y_x the test is impossible.

Unplanned carriers form a second group. An unauthorized carrier, a carrier at the wrong frequency and adjacent-satellite interference all add energy outside the expected occupancy, and their spectra can be nearly identical. What separates them is whether the energy is consistent with the operator's carrier plan, and where it originates. CID answers the second question when the transmitter cooperates [13], [14]. Without CID or geolocation, adjacent-satellite and unauthorized carriers may be inseparable from the downlink alone, and a recognizer should report that confusion rather than hide it.

Carrier-under-carrier interference is the clearest identifiability limit. In paired carrier multiple access (PCMA), two carriers deliberately share one band, and blind detection of the weaker one is a known hard problem [66]; information-theoretic analysis bounds what blind detection can achieve [67]. Unintended carrier-under-carrier events inherit this difficulty, which is why commercial tools cancel the host carrier before looking for a residual [15]–[17]. At realistic carrier-to-interference ratios the composite power spectrum is essentially that of the host. Features that respond to a second symbol rate, such as the cyclic spectrum [11], [43], are the natural candidates for making this class observable.

The difficulty is visible in second-order statistics. With a host carrier s_h, an under-carrier s_u at power ratio ρ = (C/I)⁻¹ and independent stationary noise,

y(t) = s_h(t) + \sqrt{\rho}\, s_u(t) + n(t) \quad\Longrightarrow\quad S_y(f) = S_h(f) + \rho\, S_u(f) + N_0

so at C/I = 20 dB and equal bandwidths the under-carrier raises the power spectral density by only 10 log₁₀(1 + 0.01) ≈ 0.04 dB. The spectral correlation function

S_y^{\alpha}(f) = \lim_{T \to \infty} \frac{1}{T}\, \mathbb{E}\left[ Y_T\!\left(f + \tfrac{\alpha}{2}\right) Y_T^{*}\!\left(f - \tfrac{\alpha}{2}\right) \right]

is non-zero only at the cycle frequencies α = n/T_s of each linearly modulated component, and vanishes for stationary noise [11]. When the symbol periods differ, S_y^α at α = 1/T_u contains only the under-carrier term ρ S_u^α, free of the host's much larger power. When they coincide, the cyclic signatures overlap as well, and only cancellation of the decoded host separates the two [15]–[17].

Finally, payload intermodulation must be separated from external interference. Here the transponder physics helps: intermodulation products fall at frequencies set by the carrier plan and change with input back-off in a predictable way [55], [58], whereas external interferers follow neither rule. A recognizer that knows the carrier plan and sees behavior across drive levels can in principle tell the two apart; one that sees a single unannotated spectrum cannot. The IMUX/OMUX masking effect [56] complicates this, since the observed signature depends on the filters as well as the amplifier.

We found no paper that formalizes these arguments as an identifiability analysis for satellite interference classes; the closest formal treatment is the information-theoretic study of blind PCMA detection [67]. Mapping each class to the observables it requires — co-polar spectrum, cross-polar spectrum, carrier plan, response to back-off, cyclic features — is therefore open, and contribution C2 addresses it directly.

Table 1 states this argument as a working hypothesis; the ablations in Section 7 are designed to test each entry.

Table 1. Observables expected to be required (●), helpful (○) or uninformative (–) for each known class.

| Class | Co-pol spectrogram | Cross-pol spectrogram | Carrier-plan mask | Response to IBO | Cyclic features |
| --- | --- | --- | --- | --- | --- |
| Clean | ● | ○ | ● | ○ | ○ |
| CW | ● | – | ○ | – | ○ |
| Swept CW | ● | – | ○ | – | – |
| Cross-pol leakage | ○ | ● | ○ | – | – |
| Adjacent-satellite | ● | – | ● | – | ○ |
| Unauthorized carrier | ● | – | ● | – | ○ |
| Carrier-under-carrier | ○ | – | ○ | – | ● |
| Overdrive IM | ● | – | ● | ● | – |

Adjacent-satellite and unauthorized carriers share the same row of observables, which is the formal sense in which they may be inseparable from the downlink alone. Carrier-under-carrier is the only class whose required observable is cyclic, and overdrive IM is the only one that needs the response to back-off.

## 6. Open-set recognition

A recognizer trained on a fixed list of classes will, by default, assign every input to one of them. Open-set recognition (OSR) and out-of-distribution (OOD) detection address this by allowing a model to reject inputs that belong to no known class [68], [69]. For a GEO operator, rejection is not an edge case: about a third of logged incidents historically came from sources that could not be identified [1], and new forms of jamming continue to appear [5].

The methods most relevant here are post-hoc scores computed on a trained classifier. Maximum softmax probability (MSP) is the standard baseline [70], and ODIN sharpens it with temperature scaling and input perturbation [71]. The energy score replaces the softmax with a log-sum-exp of the logits and separates in- and out-of-distribution data more reliably [72]. The Mahalanobis score fits class-conditional Gaussians to the network's embeddings and measures distance to the nearest class [73]. Training-time methods such as OpenMax [74] and adversarial reciprocal points [75] reshape the representation itself. A result with direct consequences for experimental design is that a well-trained closed-set classifier, scored with MSP or maximum logit, matches or beats many specialized OSR methods [76]. Open-set performance is therefore tied to closed-set accuracy, and the open-set classification rate (OSCR) [77] measures both together.

All three post-hoc scores fit one decision rule. With logits f_k(x), embedding z = z(x), and class means μ_k and shared covariance Σ estimated on training data, a threshold τ on a score S(x) turns the classifier into an open-set recognizer:

\hat{y}(x) = \begin{cases} \arg\max_k f_k(x), & S(x) \ge \tau \\ \text{unknown}, & S(x) < \tau \end{cases}

S_{\mathrm{MSP}}(x) = \max_k \frac{e^{f_k(x)}}{\sum_{j} e^{f_j(x)}}, \qquad S_{\mathrm{E}}(x) = T_E \log \sum_{k} e^{f_k(x)/T_E}, \qquad S_{\mathrm{M}}(x) = -\min_k \left(z - \mu_k\right)^{\top} \Sigma^{-1} \left(z - \mu_k\right)

OSCR sweeps τ and traces the correct classification rate on known data D_K against the false-positive rate on unknown data D_U [77]:

\mathrm{CCR}(\tau) = \frac{\left|\{x \in \mathcal{D}_K : \hat{y}(x) = y,\ S(x) \ge \tau\}\right|}{|\mathcal{D}_K|}, \qquad \mathrm{FPR}(\tau) = \frac{\left|\{x \in \mathcal{D}_U : S(x) \ge \tau\}\right|}{|\mathcal{D}_U|}, \qquad \mathrm{OSCR} = \int_0^1 \mathrm{CCR}\; d\,\mathrm{FPR}

Because CCR counts only correctly classified known samples, OSCR falls when either closed-set accuracy or rejection degrades, which is why it complements AUROC.

These methods have spread widely through RF recognition. Open-set automatic modulation classification has been addressed with LSTM models and extreme-value calibration [78], reconstruction-based rejection [79], few-shot formulations [80] and studies of adversarial robustness [81]. Open-set RF fingerprinting uses prototype learning [82], and pairing a deep model with an expert-feature classifier improves open-set wireless signal classification [83]. Jamming recognition has followed: convolutional prototype learning rejects unknown classes among nine simulated jamming types [84], OpenMax and extreme-value methods handle unknown radar jamming [85], [86], and domain-specific networks and prototype methods continue the line [87], [88]. In GNSS, few-shot learning for new interference classes [34] and uncertainty-based reliability [32] pursue related goals.

None of this work concerns satellite transponders. The satellite interference models of Section 3 are either anomaly detectors, which flag deviations but cannot name known causes, or closed-set classifiers, which name causes but cannot reject unknown ones. Agon's out-of-distribution tests [24] address covariate shift — the same task on different constellations and datasets — rather than unknown interference classes. The observation that OSR has been neglected in deep-learning interference work [47] holds for the GEO case in particular.

The OSR literature also warns of evaluation pitfalls that shape our protocol. How hard the unknowns are depends on how close they lie to the known classes, so results must be reported per unknown class rather than pooled [69], [76]. In our class set, wideband noise is spectrally close to overdrive intermodulation regrowth, and intermittent bursts resemble pulsed interference. AUROC alone hides the operating point, so OSCR [77] and the false-positive rate at 95% true-positive rate are reported alongside it. Thresholds and score parameters must never be tuned on the test unknowns.

## 7. Evaluation under distribution shift

A simulator-trained recognizer will meet operating conditions it was not trained on, and the RF literature shows that this matters. Over-the-air RadioML experiments found that models trained in simulation lose accuracy on real captures [39]. Fraunhofer's GNSS evaluations found that supervised models degrade under discrepancies between controlled and field recordings, and that unsupervised and domain-adaptation methods recover only part of the loss [35]. Embedding-based domain generalization [89] and synthetic-to-real tests of RF vision-language models [90] reach the same conclusion: without calibration or adaptation, performance drops on shifted data.

Satellite interference studies have only begun to measure this. The TMLCN detectors were tested across modulation orders, bandwidth overlaps and off-axis angles [23], and Agon reports cross-constellation and cross-dataset robustness [24]. No study varies the payload operating point — input back-off, transponder bandwidth or carrier loading — or measures what a model loses when trained without the nonlinear chain. That measurement is contribution C3. It follows the GNSS practice of splitting by condition rather than at random [35]: test sets hold out back-off values, transponder bandwidths (36 MHz and 72 MHz) and loading levels unseen in training, so that reported numbers estimate generalization rather than interpolation.

We define the payload gap at a test condition c as the difference in a metric M between a model trained with the full transponder chain and one trained on linearly superposed signals, both evaluated on full-chain test data D_c:

\Delta_{\mathrm{payload}}(c) = M\!\left(f_{\mathrm{full}};\, \mathcal{D}_c\right) - M\!\left(f_{\mathrm{lin}};\, \mathcal{D}_c\right)

Plotted against the distance between c and the training conditions (for example |IBO_test − IBO_train|), Δ_payload shows how the value of payload modeling grows as the operating point moves away from what the model saw.

Random splits of simulated RF data also invite leakage. If the same carrier-plan instance, payload random seed or normalization statistics appear in both training and test data, a model can score well by memorizing scenario-specific features. Condition-based splits, per-split normalization and separate seeds per partition guard against this.

The metrics follow from the operator's use. Detection probability at a fixed false-alarm rate, plotted against carrier-to-interference ratio, is the standard figure of merit in the detection literature [8], [9], [23]. Macro-F1 on known classes, AUROC and OSCR [77] on unknowns, and calibration error cover recognition and rejection. Parameter count, FLOPs and per-frame latency matter because monitoring systems sweep many carriers continuously — one vendor advertises up to 200 carriers per second [16] — and because compact models are known to suffice for interference tasks [28], [45]. Baselines include CA-CFAR [10], expert spectral and cyclostationary features with gradient boosting [11], [91], signal-cancellation energy detection [8], and a reconstruction detector of the TrID type [23].

For a detection statistic T and K known classes, the two headline metrics are

P_{\mathrm{D}}(P_{\mathrm{FA}}) = \Pr\{T > \gamma \mid \mathcal{H}_1\}, \qquad \gamma : \Pr\{T > \gamma \mid \mathcal{H}_0\} = P_{\mathrm{FA}}

\mathrm{macro\text{-}F_1} = \frac{1}{K} \sum_{k=1}^{K} \frac{2\, \mathrm{Prec}_k\, \mathrm{Rec}_k}{\mathrm{Prec}_k + \mathrm{Rec}_k}

Macro averaging weights rare classes such as carrier-under-carrier equally with common ones, so a model cannot score well by ignoring them.

Public data supports these checks without replacing operator recordings. SnT's on-board and NGSO-to-GSO datasets [92], [93] allow direct comparison with prior GEO work, though they are released for non-commercial research only and contain no payload effects. Fraunhofer's GNSS recordings [94], [95], released under Creative Commons licenses, supply real pulsed, chirp and bursty interferers for testing unknown-class rejection. TorchSig [40], [41] supports pre-training and waveform generation, and OpenDPD [61] supports amplifier-mismatch tests.

## 8. Positioning of this work

The literature contains the pieces of a payload-aware, open-set interference recognizer, but in separate fields that have not been combined. Satellite payload modeling is mature but serves compensation [53]–[56]. Satellite interference learning is active but uses linear, single-carrier, single-polarization signal models with binary or closed-set tasks [20]–[24]. Open-set recognition is mature and widely applied to RF signals, but not to satellite transponders [76], [84]. The table shows where the closest prior work stands on the properties this project targets.

| Work | Payload nonlinearity | IMUX/OMUX | Multicarrier IM | Dual polarization | Carrier plan | Unknown-class rejection | Data |
| --- | --- | --- | --- | --- | --- | --- | --- |
| On-board detection with cancellation [8], [9] | No | No | No | No | No | No | Analytical / simulated |
| Autoencoder and LSTM detectors [18], [19] | No | No | No | No | No | No | Real Ku-band spectra |
| Operator ML deployment [6] | No | No | No | Motivation only | No | No | Real data |
| On-board CNN [20] | No | No | No | No | Partial (subband grid) | No | Simulated |
| NGSO-to-GSO detectors [21]–[23] | No | No | No | No | No | No | Simulated, public |
| Agon [24] | Testbed only | No | No | No | No | Covariate shift only | Public + testbed |
| Closed-set classifiers [25]–[27] | No | No | No | No | No | No | Simulated AWGN |
| Open-set jamming recognition [84]–[87] | No | No | No | No | No | Yes | Simulated |
| GNSS interference ML [32]–[35] | No | No | No | No | No | Partial (novel classes) | Real + controlled |
| DVB-S2X reference channel [54] | Yes | Yes | Yes | Assumed negligible | Partial | Not applicable | Reference model |
| This project | Yes | Yes | Yes | Yes | Yes | Yes | Simulated + realism checks |

Table 2 collects the results the closest studies report. No two rows share a dataset, a task and a metric, so the numbers cannot be ranked against one another; they show what has been claimed and under which signal model.

Table 2. Reported results of the closest prior work, as stated in each source.

| Work | Task | Data | Signal model | Reported result |
| --- | --- | --- | --- | --- |
| Vázquez et al. [6] | Binary detection, autoencoder | Real data, operator context | Real captures | 44% lower false-detection probability than a fixed-threshold baseline |
| Pellaco et al. [19] | Detection by spectrum prediction | Real spectra, Swedish Space Corporation | Real captures | More reliable than a model-based baseline; no single figure given |
| Daoud et al. [20] | Subband localization, on-board CNN | Simulated | BPSK, linear superposition | Reduced-dimension FFT magnitude gives the best accuracy |
| Saifaldawla et al. [22] | Binary detection, convolutional autoencoders | Simulated NGSO→GSO | Linear superposition | Up to 11% higher detection probability than energy detection |
| Saifaldawla et al. [23] | Binary detection, VAE and transformer (TrID) | Public NGSO–GSO set | Linear superposition | Up to 31.23% higher detection accuracy than energy detection |
| Yang et al. (Agon) [24] | Binary detection, semi-supervised transformer | NGSO–GSO and NGSO–NGSO sets | Linear superposition | AUC 0.9327, F1 0.8351 (NGSO–GSO); AUC 0.9085, F1 0.9055 (NGSO–NGSO) |
| Chen et al. [84] | Open-set jamming recognition | Simulated, nine jamming types | Generic jamming in noise | Unknown-rejection plateau 0.9883 at openness 0.030; above 0.89 at 0.397 |
| Ouyang [87] | Open-set jamming recognition | Simulated compound jamming | Generic jamming | 98.22% closed-set accuracy |
| Liu et al. [88] | Open-set radar jamming recognition | Simulated | Radar jamming | Unknown-jamming recognition above 90% at JSR 5 dB, SNR above 0 dB |
| Ott et al. [94] | GNSS interference classification | Real highway recordings, 8 interference + 3 clean classes | Real captures | 97.66% baseline accuracy |

Two patterns stand out. The satellite rows report detection, not recognition, and every simulated satellite row uses linear superposition. The open-set rows report rejection of unknown jamming, but none involves a satellite payload.

Against this background, the three candidate contributions can be stated precisely.

C1 — a payload-realistic, open-source data generator. It combines what the reference channel models — multicarrier DVB-S2 loading, TWTA AM/AM–AM/PM over an IBO sweep, IMUX/OMUX filtering, phase noise [53], [54] — with what it omits: cross-polarization, carrier-plan metadata and a labelled interference catalogue. Following open dataset practice in GNSS [37], it should ship with fixed benchmark splits and randomized payload parameters.

C2 — a plan-conditioned, dual-polarization open-set recognizer with an identifiability analysis. Section 5 shows why each input is needed: cross-polarization requires the second polarization, unplanned-carrier and intermodulation classes require the plan, and carrier-under-carrier requires cyclic features. This mapping of classes to required observables has not been done for satellite interference.

C3 — a quantified payload gap. No study measures how much a recognizer trained without the transponder chain loses when deployed behind one, or how performance falls with distance from the training back-off, bandwidth and loading. Reporting this as a curve rather than a single number would give operators and later studies a concrete measure of what simulation fidelity buys.

Three limits of the literature should be stated alongside these claims. Agon [24], the Eutelsat/CTTC operational work [6] and open-set jamming recognition [84] each overlap part of the problem, so the novelty lies in the joint treatment rather than in any single element. All comparable GEO studies are simulation-based, so realism must be demonstrated through traceability to the DVB-S2X reference channel [54], measured amplifier data [61] and comparison with public datasets [92], [93]. And the interference statistics that motivate the work come from industry sources [1]–[3], not from peer-reviewed measurement.

## References

Every entry was checked against Crossref, arXiv or DataCite, or against the publisher's page; the reference manifest records the evidence for each. Trade-press and vendor sources [1]–[3], [5], [14]–[17] are cited only for operator statistics and product capabilities and are not peer-reviewed.

- [1] J. Bates, "Satellite operators target interference problem," Via Satellite, 2005. Link

- [2] M. Coleman, "Tackling satellite interference," Microwave Journal, 2012. Link

- [3] Kratos, "Complete VSAT interference mitigation," fact sheet. Link

- [4] M. Á. Vázquez, P. Henarejos, A. I. Pérez-Neira, E. Grechi, A. Voight, J. C. Gil, I. Pappalardo, F. Di Credico, R. M. Lancellotti, "On the use of AI for satellite communications," IEEE ComSoc SSC Newsletter, 2019, arXiv:2007.10110. Open access

- [5] Kratos, "Kratos KnownSpace detects sustained Iran origin RF interference targeting GEO communications," press release, 2026. Link

- [6] M. Á. Vázquez, P. Henarejos, I. Pappalardo, E. Grechi, J. Fort, J. C. Gil, R. M. Lancellotti, "Machine learning for satellite communications operations," IEEE Commun. Mag., vol. 59, no. 2, pp. 22-27, 2021, doi: 10.1109/MCOM.001.2000367. DOI

- [7] A. I. Pérez-Neira, M. Á. Vázquez, S. Maleki, M. R. B. Shankar, S. Chatzinotas, "Signal processing for high throughput satellite systems: Challenges in new interference-limited scenarios," arXiv:1802.03958, 2018. Open access

- [8] C. Politis, S. Maleki, C. Tsinos, S. Chatzinotas, B. Ottersten, "On-board the satellite interference detection with imperfect signal cancellation," Proc. IEEE SPAWC, pp. 1-5, 2016, doi: 10.1109/SPAWC.2016.7536813. DOI

- [9] C. Politis, S. Maleki, C. Tsinos, S. Chatzinotas, B. Ottersten, "Weak interference detection with signal cancellation in satellite communications," Proc. IEEE ICASSP, pp. 6289-6293, 2017, doi: 10.1109/ICASSP.2017.7953366. Link

- [10] H. M. Finn, R. S. Johnson, "Adaptive detection mode with threshold control as a function of spatially sampled clutter-level estimates," RCA Review, vol. 29, no. 3, pp. 414-464, 1968.

- [11] W. A. Gardner, "Exploitation of spectral redundancy in cyclostationary signals," IEEE Signal Process. Mag., vol. 8, no. 2, pp. 14-36, 1991, doi: 10.1109/79.81007. DOI

- [12] B. Kim, H. Yu, S. Noh, "Cognitive interference cancellation with digital channelizer for satellite communication," Sensors, vol. 20, no. 2, art. 355, 2020, doi: 10.3390/s20020355. Link

- [13] ITU-R, "Carrier identification system for FSS occasional use earth station transmissions," Recommendation S.2062-0, 2014. Link

- [14] B. Potter, "Preventing adjacent satellite interference," Via Satellite, 2014. Link

- [15] Rohde & Schwarz, "R&S GSACSM communications system monitoring," product page. Link

- [16] Integrasys, "VectorSat interference detection," product page. Link

- [17] Calian, "Decimator carrier-under-carrier detection," product page. Link

- [18] P. Henarejos, M. Á. Vázquez, A. I. Pérez-Neira, "Deep learning for experimental hybrid terrestrial and satellite interference management," Proc. IEEE SPAWC, pp. 1-5, 2019, doi: 10.1109/SPAWC.2019.8815532, arXiv:1906.03012. Open access

- [19] L. Pellaco, N. Singh, J. Jaldén, "Spectrum prediction and interference detection for satellite communications," Proc. ICSSC, 2019, arXiv:1912.04716. Open access

- [20] S. Daoud, G. Eappen, F. Ortiz, E. Lagunas, W. Martins, S. Chatzinotas, "CNN-based on-board interference detection in satellite systems: An analysis of dataset impact on performance," Proc. IEEE ICASSP Workshops (ICASSPW), pp. 1-5, 2023, doi: 10.1109/ICASSPW59220.2023.10193395. Link

- [21] A. Saifaldawla, F. Ortiz, E. Lagunas, S. Daoud, S. Chatzinotas, "NGSO-to-GSO satellite interference detection based on autoencoder," Proc. IEEE PIMRC, pp. 1-7, 2023, doi: 10.1109/PIMRC56721.2023.10293839. Link

- [22] A. Saifaldawla, F. Ortiz, E. Lagunas, S. Chatzinotas, "Convolutional autoencoders for non-geostationary satellite interference detection," Proc. IEEE ICC Workshops, pp. 1334-1339, 2024, doi: 10.1109/ICCWorkshops59551.2024.10615457. Link

- [23] A. Saifaldawla, F. Ortiz, E. Lagunas, A. B. M. Adam, S. Chatzinotas, "GenAI-based models for NGSO satellites interference detection," IEEE Trans. Mach. Learn. Commun. Netw., vol. 2, pp. 904-924, 2024, doi: 10.1109/TMLCN.2024.3418933. DOI

- [24] B. Yang, C. Yang, Z. Chen, K. Qiu, Y. Gao, "Agon: A semi-supervised framework for robust satellite interference detection," arXiv:2606.14147 (preprint), 2026. Open access

- [25] H. Yu, R. Zhang, R. Ding, "Interference recognition based on machine learning for satellite communications," ACM conference proceedings, pp. 18-23, 2018, doi: 10.1145/3231884.3231901. DOI

- [26] X. Han, "Artificial intelligence-based interference signal recognition for satellite communications," Proc. ACSAT, pp. 279-282, 2024, doi: 10.1109/ACSAT63853.2024.10823708. Link

- [27] H. Xiong, "Research on interference detection and identification algorithms for satellite communication systems," Proc. ICECAI, pp. 84-87, 2025, doi: 10.1109/ICECAI66283.2025.11170987. Link

- [28] F. Ortiz, V. Monzon Baeza, L. M. Garces-Socarras, J. A. Vásquez-Peralvo, J. L. Gonzalez et al., "Onboard processing in satellite communications using AI accelerators," Aerospace, vol. 10, no. 2, art. 101, 2023, doi: 10.3390/aerospace10020101. Link

- [29] F. Fourati, M.-S. Alouini, "Artificial intelligence for satellite communication: A review," arXiv:2101.10899, 2021. Open access

- [30] G. Fontanesi, F. Ortíz, E. Lagunas et al., "Artificial intelligence for satellite communication: A survey," preprint, Univ. Luxembourg ORBilu. Link

- [31] A. Bhattacharyya, S. M. Nambiar, R. Ojha, A. Gyaneshwar, U. Chadha, K. Srinivasan, "Machine learning and deep learning powered satellite communications: Enabling technologies, applications, open challenges, and future research directions," Int. J. Satell. Commun. Netw., vol. 41, no. 6, pp. 539-588, 2023, doi: 10.1002/sat.1482. DOI

- [32] T. Brieger, N. L. Raichur, D. Jdidi, F. Ott, T. Feigl, J. R. van der Merwe et al., "Multimodal learning for reliable interference classification in GNSS signals," Proc. ION GNSS+, pp. 3210-3234, 2022. Link

- [33] J. R. van der Merwe et al., "Low-cost COTS GNSS interference monitoring, detection, and classification system," Sensors, vol. 23, no. 7, art. 3452, 2023, doi: 10.3390/s23073452. DOI

- [34] F. Ott, L. Heublein, N. L. Raichur, T. Feigl, J. Hansen, A. Rügamer, C. Mutschler, "Few-shot learning with uncertainty-based quadruplet selection for interference classification in GNSS data," Proc. ICL-GNSS, pp. 1-7, 2024, doi: 10.1109/ICL-GNSS60721.2024.10578525. DOI

- [35] L. Heublein, N. L. Raichur, T. Feigl, T. Brieger, F. Heuer et al., "Evaluation of (un-)supervised machine learning methods for GNSS interference classification with real-world data discrepancies," Proc. ION GNSS+, 2024, arXiv:2503.23775. Open access

- [36] L. Heublein et al., "VAE-based feature disentanglement for data augmentation and compression in generalized GNSS interference classification," arXiv:2504.10556, 2025. Open access

- [37] C. Wielenberg, L. Heublein, J. Ott et al., "The S-ICDF dataset: Sionna-simulated dynamic interference characterization and direction finding," arXiv:2607.03411, 2026. Open access

- [38] I. Ebrahimi Mehr, F. Dovis, "A deep neural network approach for classification of GNSS interference and jamming," IEEE Trans. Aerosp. Electron. Syst., vol. 61, no. 2, pp. 1660-1676, 2025, doi: 10.1109/TAES.2024.3462662. Link

- [39] T. J. O'Shea, T. Roy, T. C. Clancy, "Over-the-air deep learning based radio signal classification," IEEE J. Sel. Topics Signal Process., vol. 12, no. 1, pp. 168-179, 2018, doi: 10.1109/JSTSP.2018.2797022, arXiv:1712.04578. Open access

- [40] L. Boegner, M. Gulati, G. Vanhoy, P. Vallance, B. Comar, S. Kokalj-Filipovic, C. Lennon, R. D. Miller, "Large scale radio frequency signal classification," arXiv:2207.09918, 2022. Open access

- [41] L. Boegner et al., "Large scale radio frequency wideband signal detection & recognition," arXiv:2211.10335, 2022. Open access

- [42] M. Kulin, T. Kazaz, I. Moerman, E. De Poorter, "End-to-end learning from spectrum data: A deep learning approach for wireless signal identification in spectrum monitoring applications," IEEE Access, vol. 6, pp. 18484-18501, 2018, doi: 10.1109/ACCESS.2018.2818794, arXiv:1712.03987. Open access

- [43] K. Tekbıyık, Ö. Akbunar, A. R. Ekti, A. Görçin, G. K. Kurt, "Spectrum sensing and signal identification with deep learning based on spectral correlation function," arXiv:2003.08359, 2020. Open access

- [44] R. Wu, C. J. Li, W. Zhang, X. Liu, P. H. W. Leong, "Cyclo-AMC: Automatic modulation classification on Versal utilising cyclostationary features," Proc. FPT, 2025. Link

- [45] T. Oyedare, V. K. Shah, D. J. Jakubisin, J. H. Reed, "Keep it simple: CNN model complexity studies for interference classification tasks," arXiv:2303.03326, 2023. Open access

- [46] C. Xu, J. Ren, W. Yu, Y. Jin, Z. Cao, X. Wu, W. Jiang, "Time-frequency analysis based deep interference classification for frequency hopping system," arXiv:2108.10056, 2021. Open access

- [47] T. Oyedare, V. K. Shah, D. J. Jakubisin, J. H. Reed, "Interference suppression using deep learning: Current approaches and open challenges," arXiv:2112.08988, 2021. Open access

- [48] K. He, X. Zhang, S. Ren, J. Sun, "Deep residual learning for image recognition," Proc. CVPR, 2016, arXiv:1512.03385. Open access

- [49] Y. Gong, Y.-A. Chung, J. Glass, "AST: Audio spectrogram transformer," Proc. Interspeech, 2021, arXiv:2104.01778. Open access

- [50] E. Perez, F. Strub, H. de Vries, V. Dumoulin, A. Courville, "FiLM: Visual reasoning with a general conditioning layer," Proc. AAAI, 2018, arXiv:1709.07871. Open access

- [51] A. A. M. Saleh, "Frequency-independent and frequency-dependent nonlinear models of TWT amplifiers," IEEE Trans. Commun., vol. 29, no. 11, pp. 1715-1720, 1981, doi: 10.1109/TCOM.1981.1094911. DOI

- [52] A. Morello, V. Mignone, "DVB-S2: The second generation standard for satellite broad-band services," Proc. IEEE, vol. 94, no. 1, pp. 210-227, 2006, doi: 10.1109/JPROC.2005.861013. DOI

- [53] E. Casini, R. De Gaudenzi, A. Ginesi, "DVB-S2 modem algorithms design and performance over typical satellite channels," Int. J. Satell. Commun. Netw., vol. 22, no. 3, pp. 281-318, 2004, doi: 10.1002/sat.791. DOI

- [54] ETSI/DVB, "DVB-S2X implementation guidelines," TR 102 376-2 (DVB BlueBook A171-2), 2020. Link

- [55] B. F. Beidas, "Intermodulation distortion in multicarrier satellite systems: Analysis and turbo Volterra equalization," IEEE Trans. Commun., vol. 59, no. 6, pp. 1580-1590, 2011, doi: 10.1109/TCOMM.2011.042111.100320. Link

- [56] M. Allegue-Martínez et al., "Digital linear pre-compensation technique to enhance predistortion performance in multicarrier DVB-S2 satellite communication systems," Electron. Lett., vol. 50, no. 13, pp. 957-959, 2014, doi: 10.1049/el.2014.1172. DOI

- [57] A. A. Gözütok, M. M. Nefes, S. Demirel, "NPR and intermodulation distortion modelling of travelling-wave tube amplifier of telecommunication spacecraft," 2021. Link

- [58] J. C. Fuenzalida, O. Shimbo, W. L. Cook, "Time-domain analysis of intermodulation effects caused by nonlinear amplifiers," COMSAT Tech. Rev., vol. 3, no. 1, pp. 89-143, 1973.

- [59] Y. Luo, A. Jayaprakash, G. Chen, C. Huang, Q. Luo, P. Xiao, "Widely linear augmented extreme learning machine based impairments compensation for satellite communications," arXiv:2506.14557, 2025. Open access

- [60] X. Sun et al., "Kernel-FastICA-based nonlinear blind source separation for anti-jamming satellite communications," Sensors, vol. 25, no. 12, art. 3743, 2025, doi: 10.3390/s25123743. Link

- [61] Y. Wu, G. D. Singh, M. Beikmirza, L. C. N. de Vreede, M. Alavi, C. Gao, "OpenDPD: An open-source end-to-end learning & benchmarking framework for wideband power amplifier modeling and digital pre-distortion," Proc. IEEE ISCAS, 2024, arXiv:2401.08318. Open access

- [62] Y. Wu, A. Li, C. Gao, "OpenDPDv2: A unified learning and optimization framework for neural network digital predistortion," arXiv:2507.06849, 2025. Open access

- [63] S. Hanna, D. Čabrić, "Deep learning based transmitter identification using power amplifier nonlinearity," arXiv:1811.04521, 2018. Open access

- [64] A. S. Karasuwa, J. D. Eastment, I. E. Otung, "Cross-polarisation discrimination-induced interference in dual-polarised high-capacity satellite communication systems," J. Eng., vol. 2016, no. 6, pp. 162-170, 2016, doi: 10.1049/joe.2015.0178. Link

- [65] E. Satorius, T. Jedrey, C. Wang, "Impact of cross-polarization interference on dual polarization communication systems," IPN Progress Report 42-213, JPL, 2018. Link

- [66] P. Mo, X. Fan, L. You, "A novel and blind detection scheme for asymmetric PCMA system," Proc. ICCSNT, pp. 854-857, 2013, doi: 10.1109/ICCSNT.2013.6967240. Link

- [67] X. Liu, X. Tao, X. Chen, N. Ge, "Information theory analysis of blind detection for PCMA satellite communication systems," Proc. IEEE VTC-Fall, pp. 1-5, 2013, doi: 10.1109/VTCFall.2013.6692242. Link

- [68] C. Geng, S.-J. Huang, S. Chen, "Recent advances in open set recognition: A survey," IEEE Trans. Pattern Anal. Mach. Intell., vol. 43, no. 10, pp. 3614-3631, 2021, doi: 10.1109/TPAMI.2020.2981604, arXiv:1811.08581. Open access

- [69] J. Yang, K. Zhou, Y. Li, Z. Liu, "Generalized out-of-distribution detection: A survey," Int. J. Comput. Vis., vol. 132, no. 12, pp. 5635-5662, 2024, doi: 10.1007/s11263-024-02117-4, arXiv:2110.11334. Open access

- [70] D. Hendrycks, K. Gimpel, "A baseline for detecting misclassified and out-of-distribution examples in neural networks," Proc. ICLR, 2017, arXiv:1610.02136. Open access

- [71] S. Liang, Y. Li, R. Srikant, "Enhancing the reliability of out-of-distribution image detection in neural networks," Proc. ICLR, 2018, arXiv:1706.02690. Open access

- [72] W. Liu, X. Wang, J. D. Owens, Y. Li, "Energy-based out-of-distribution detection," Proc. NeurIPS, 2020, arXiv:2010.03759. Open access

- [73] K. Lee, K. Lee, H. Lee, J. Shin, "A simple unified framework for detecting out-of-distribution samples and adversarial attacks," Proc. NeurIPS, 2018, arXiv:1807.03888. Open access

- [74] A. Bendale, T. Boult, "Towards open set deep networks," Proc. CVPR, 2016, arXiv:1511.06233. Open access

- [75] G. Chen, P. Peng, X. Wang, Y. Tian, "Adversarial reciprocal points learning for open set recognition," IEEE Trans. Pattern Anal. Mach. Intell., 2021, doi: 10.1109/TPAMI.2021.3106743, arXiv:2103.00953. Open access

- [76] S. Vaze, K. Han, A. Vedaldi, A. Zisserman, "Open-set recognition: A good closed-set classifier is all you need?," Proc. ICLR, 2022, arXiv:2110.06207. Open access

- [77] A. R. Dhamija, M. Günther, T. E. Boult, "Reducing network agnostophobia," Proc. NeurIPS, 2018, arXiv:1811.04110. Open access

- [78] Y. Guo, H. Jiang, J. Wu, J. Zhou, "Open set modulation recognition based on dual-channel LSTM model," arXiv:2002.12037, 2020. Open access

- [79] Z. Zhang, M. Zhu, J. Liu, Y. Li, S. Wang, "Class information guided reconstruction for automatic modulation open-set recognition," arXiv:2312.13023, 2023. Open access

- [80] H. Zhang, F. Zhou, Q. Wu, C. Yuen, "FSOS-AMC: Few-shot open-set learning for automatic modulation classification," arXiv:2410.10265, 2024. Open access

- [81] Y. Yang, S. Zhang, K. Li, Q. Tian, Y. Lin, "Adversarial threats to automatic modulation open set recognition in wireless networks," arXiv:2405.05022, 2024. Open access

- [82] W. Wang, H. Liao, L. Gan, "Open-set RF fingerprinting via improved prototype learning," arXiv:2306.13895, 2023. Open access

- [83] S. R. Shebert, B. H. Kirk, R. M. Buehrer, "Open set wireless signal classification: Augmenting deep learning with expert feature classifiers," arXiv:2302.03749, 2023. Open access

- [84] X. Chen, Z. Zhao, X. Ye, S. Zheng, C. Lou, X. Yang, "Efficient open-set recognition for interference signals based on convolutional prototype learning," Appl. Sci., vol. 12, no. 9, art. 4380, 2022, doi: 10.3390/app12094380. DOI

- [85] Y. Zhou, S. Shang, X. Song, S. Zhang, T. You et al., "Intelligent radar jamming recognition in open set environment based on deep learning networks," Remote Sens., vol. 14, no. 24, art. 6220, 2022, doi: 10.3390/rs14246220. DOI

- [86] Y. Xiao, R. Zhang, X. Yu, Y. Jiang, "Open-set recognition of compound jamming signal based on multi-task multi-label learning," IET Radar Sonar Navig., vol. 18, no. 8, pp. 1235-1246, 2024, doi: 10.1049/rsn2.12561. DOI

- [87] M. Ouyang, "An open-set jamming signal recognition method based on domain-specific network," IET Commun., vol. 20, no. 1, 2026, doi: 10.1049/cmu2.70198. DOI

- [88] Y. Liu, S. Han, Z. Lin, C. Guo, Q. Zhao, W. Chen, T. Qiu, "An open-set recognition method for active radar jamming signals based on multi-dimensional prototype feature constraints and adversarial reciprocal point learning," Preprints.org (not peer-reviewed), 2025, doi: 10.20944/preprints202503.2079.v1. Link

- [89] L. Henneke, F. Kurth, "Approaching domain generalization with embeddings for robust discrimination and recognition of RF communication signals," arXiv:2510.23186, 2025. Open access

- [90] A. Bara, L. Bariah, H. Zou, B. Mefgouda, M. Debbah, "RF-Analyzer: Can vision-language models learn RF understanding from synthetic data?," arXiv:2605.04676, 2026. Open access

- [91] T. Chen, C. Guestrin, "XGBoost: A scalable tree boosting system," Proc. ACM KDD, 2016, arXiv:1603.02754. Open access

- [92] SnT SmartSpace, "Satellite on-board interference detection dataset," dataset (non-commercial research use). Link

- [93] SnT SmartSpace, "NGSO to GSO's users interference dataset," dataset (non-commercial research use). Link

- [94] F. Ott, L. Heublein, N. Raichur, T. Feigl, A. Rügamer, "GNSS spectrum highway dataset 1," IEEE DataPort, CC BY 4.0, 2024, doi: 10.21227/xpm9-tt28. Link

- [95] Fraunhofer IIS, "Highway 2, LC Laboratory and Jammertest 2025 GNSS interference datasets," Zenodo, CC BY-SA 4.0. Highway 2 · LC Laboratory · Jammertest 2025